# Gulf Coast Weatherization Scenario

- Property Characteristics

- Two bedroom, single family circa 1950

- Wood frame with asphalt roof

- Single story with slab on grade

- Original, single pane, double hung window sashes

- Weatherization Audit Findings

- 4 people live in house, two adults (mother & elderly grandmother) & two children (1 under six years old)

- Overall building conditions are fair

- Insufficient insulation in the attic – ¾” fiberglass

- Walls limited or no insulation

- Gas, forced hot air heating/cooling system, furnace filter dirty

- Duct work in attic not properly sealed

- High CO levels from old gas hot water heater

- Mold on bathroom ceiling & walls, bath fan not working

- Electric dryer with damaged vent

- Unvented space heater

- Damaged paint on old windows

- Leaking gas hot water heater in closet

- Weeds growing out of gutters & some downspouts missing

- Healthy Concerns

- One adult & one child have asthma, suspected dust allergy

- Grandmother is unsteady, walks with cane

- No working smoke or CO monitors

- Problem with cockroaches in kitchen and bedroom

- Possible Lead Based Paint on windows

- Gas range with no vent

- Carpeting is soiled in many spots

- Mold on bathroom ceiling & walls, bath fan not working

- No grab bars in bathroom – fall hazard

- Remediation Response

Weatherization Actions

- Insulate attic & walls

- Seal duct work with mastic

- Air sealing and weather strip

- Install smoke and CO alarms

- Replace gas hot water heater with efficient heater

- Provide and install CFL bulbs

Potential Health Responses

- Clean mold in bathroom & replace sheetrock if necessary

- Install timed, bathroom fan, confirm venting to exterior

- Replace dryer vent

- Change furnace filter supply MERV 8 if system can accept it

- Seal holes for roaches, and provide owner with Integrated Pest Management (IPM) advice or IPM services

- Install range hood fan vented to the exterior

- Replacement hot water heater is direct vent or power vented

- Remove old carpet, smooth cleanable flooring in one room  - may be outside scope of some programs

- Make referral to lead poisoning prevention program

- Conduct asthma education visit. Supply mattress covers, HEPA vac & other materials as needed