Saving Water in Your Multifamily Building

Apartments

- Get a flow meter bag from http://www.itseasybeinggreen.com/

  - Then you can measure flows of aerators and showers.

- Change all showerheads to 1.75 gallons per minute (gpm)

- Change kitchen sink aerators to 1.5 gpm

- Change bathroom sink aerators to 0.5 gpm

- Change toilets to dual flush toilets or high efficiency toilets with an average of 1.1-1.28 gallons per flush OR put a toilet tank bag in the back of the toilet to reduce the amount of water used to flush

- Test toilets for leaks by adding a few drops of food coloring to the tank and see if the bowl turns colors

- Install Energy Star dishwashers

Domestic Hot Water

- Use electronic mixing valves; when they fail they fail on cold

- Check setpoints regularly

- Quarterly, perform a preventive maintenance check:

  - Signs of physical deterioration: rust and corrosion

  - Failing insulation

  - System combustion efficiency

Common Areas & Exterior

- Use Energy Star washing machines

- Plant native plants so irrigation is not needed

- If irrigation

  - Drip irrigation

  - Moisture sensors

  - Clock timers

  - Weather data based controllers

  - Rainwater harvesting and graywater

- Sweep sidewalks rather than spray sidewalks

- Install low-flow aerators in any common area sinks

- For cooling towers, have a chemical management plan to avoid unnecessary bleed off

  - Install a conductivity meter to adjust the bleed rate and control concentration levels

  - Use non-potable water for makeup water

  - Submeter the makeup water

  - Design more efficient evaporative cooling systems to control evaporation and blowdown

- Check the water meter from 2-4am; if it’s turning you have a water leak

- For steam, use an infrared camera to find steam leaks

- Install waterless urinals in any public bathrooms